FAQ

Remote IP Extension between FXO and FXS

This application explains how to implement remote extension via IP, using MP-108/FXO and MP-

108/FXS gateways. In this configuration, PBX incoming calls are routed to the “Remote

Extension” via the MP-108/FXO and MP-108/FXS gateways.

Requirements:

• One MP-108/FXO gateway

• One MP-108/FXS gateway

• Analog phones (POTS)

• PBX – one or more PBX loop start lines

• LAN.

Connect the MP-108/FXO ports directly to the PBX lines as shown in the diagram below:

Dialing from Remote Extension

(Phone connected to MP-108/FXS)

􀂾 To configure the call, take these 6 steps:

1. Lift the handset to hear the dial tone coming from PBX, as if the phone was connected

directly to PBX.

2. MP-108/FXS and MP-108/FXO establish a voice path connection from the phone to the PBX

immediately the phone handset is raised.

3. Dial the destination number (the DTMF digits are sent, over IP, directly to the PBX).

4. All tones heard are generated from the PBX (such as Ringback, busy or fast busy tones).

5. There is one-to-one mapping between MP-108/FXS ports and PBX lines.

6. The call is disconnected when the phone connected to the MP-108/FXS goes onhook.

Dialing from other PBX line, or from PSTN

􀂾 To configure the call, take these 5 steps:

1. Dial the PBX subscriber number the same way as if the user’s phone was connected directly

to PBX.

2. Immediately as PBX rings into MP-108/FXO, the ring signal is “send” to phone connected to

MP-108/FXS.

3. Once the phone’s handset, connected to MP-108/FXS, is raised, the MP-108/FXO seizes the

PBX line and the voice path is established between the phone and the PBX line.

4. There is a one to one mapping between PBX lines and MP-108/FXS ports. Each PBX line is

routed to the same phone (connected to MP-108/FXS).

5. The call is disconnected when phone connected to MP-108/FXS goes onhook.

MP-108/FXS Configuration (using the Embedded Web Server)

􀂾 To configure the MP-108/FXS, take these 3 steps:

1. In the ‘Endpoint Phone Numbers’ screen, assign the phone numbers 100 to 107 for the

gateway’s endpoints.

2. In the ‘Automatic Dialing’ screen, enter the phone numbers of the MP-108/FXO gateway in

the ‘Destination Phone Number’ fields. When a phone connected to port #1 goes offhook,

the FXS gateway automatically dials the number ‘200’.

3. In the ‘Tel To IP Routing’ screen, enter 20 in the ‘Destination Phone Prefix’ field, and the IP

address of the MP-108/FXO gateway (10.1.10.2) in the field ‘IP Address’.

MP-108/FXO Configuration (using the Embedded Web Server)

􀂾 To configure the MP-108/FXO, take these 4 steps:

1. In the ‘Endpoint Phone Numbers’ screen, assign the phone numbers 200 to 207 for the

gateway’s endpoints.

2. In the ‘Automatic Dialing’ screen, enter the phone numbers of the MP-108/FXS gateway in

the ‘Destination Phone Number’ fields. When a ringing signal is detected at port #1, the FXO

gateway automatically dials the number ‘100’.

3. In the ‘Tel To IP Routing’ screen, enter 10 in the ‘Destination Phone Prefix’ field, and the IP

address of the MP-108/FXS gateway (10.1.10.3) in the field ‘IP Address’.

4. In the ‘Protocol Management’ screen, set the parameter ‘Dialing Mode’ to ‘Two Stage’

(IsTwoStageDial=1).